use manua

First time use

◊charge your headset

◊pair your headset with a bluetooth built-in mobile phone

Charging your headset

◊The bluetooth headset comes with a built-in rechargeable battery, before use this headset, please charge the battery fully for the first time use.

◊Plug AC/DC adaptor to power supplier end, then plug mini USB connector end to charge jack of headset.

Normal charging time

under normal using, battery of headset will be full after charge power at 2 hours, when battery is full, the headset will stop charge power automatically, Then constantly press the multifunction button(fig.1) at 2 seconds for turning on headset. (please refer to step of "turn on/off headset" as below.)

Charge indication

when charge power to headset, LED is red color constantly on. once battery is full, LED turns off automatically.

Talk & standby time

Under battery is charged fully and normal using, talking time up to 3 hours/standby time 100 hours.

Low battery

When the headset is on and the battery becomes low, the LED will blink red color and the headset will "beep" every 1 minute, at that time, please recharge the battery as above step as "charging your headset".

Turn on/off you headset

◊Turn on your headset

Press multifunction button for 2 seconds constantly until you hear a short beep. the LED will blink blue color.

◊Turn off the headset

Press multifunction button at 3 seconds constantly until you hear a long beep, the LED will turn off.

Pairing your headset

This headset must use with bluetooth built-in mobile phone. configuration set up as below for the first time.

1.keep headset and phone around 1 meter.

2.assure headset is turned off

3.constantly press multifunction button unti LED blinks red and blue in turn.

4.initiate bluetooth function of phones & search for bluetooth devices.

5.phone will start searching for bluetooth devices. if it is successful, phone screen will show the name of obtained device - "DKH-03". press "yes" to pair this headset to the phone.

6.input ID code of headset - "0000", press "yes" to conplete pairing.

meanwhile, this headset is under stand-by mode - blue LED blinks 5 times then having a long beep.

7.back to main manual of phone, headset is under stand-by mode and ready for bluetooth communication. please locate this headset within 10 meters(max.) to phone.

Notice:if pairing process can't complete within 2 minutes, LED will blink red color with 3 beeps. finally, headset will turn off automatically. please follow up above steps to repairing again.

Link headset with phone

Once pairing is successful, headset is under stand-by mode. Owing to different phone has different program, please refer to procedure as below.

When a call is in or dial out a call, phone will link headset automatically. if mobile phone have voice activation function, user can talk to mic. of this headset for voice dialing or redial out the last phone number.

◊When a call is in or dial out a call, phone will link headset automatically. if phone had been cut off link to headset, reconnect procedure as below.

1.press multifunction button, headset will be reconnected to phone and press "yes" on this phone to accept reconnect mode.

2.operate phone manual to bluetooth function and then into program of paired device and select "DKH-03".

◊When headset if under standby mode but can't pick up/dial out call, please follow up procedure as below for reconnect.

1.press multifunction button. headset will be reconnected to phone and press "yes" on the phone to accept reconnect mode.

2.operate phone manual to bluetooth function and then into program of paired device and select "DKH-03"

To use headset

◊To answer a call

when a call is in, a ringing tone will come out of this headset, press multifunction button to answer a call.

◊To cut off a call

under talking status, press multifunction button to cut off a call.

Voice activation

under stand-by mode, press multifunction button to enter bluetooth connection mode, user will hear a beep then headset is available to use voice activation function.

Notice

1.before using voice activation, please assure phone having built-in this function.(voice activation is subject to phones built-in with voice activation function and refer to voice dialling procedure of individual phone.)

2.under stand-by mode, press two times of multifunction button continuously, headset will dial out the last phone call.

Volume adjustment

Simply adjust the built-in volume switch to adjust volume.

To reset the headset

You may want to reset the headset to original setting, this also clears all current pairings.

1.assure the headset is off

2.constantly press the volume key until LED blinks red and blue color in turn

3.release the volume key

4.constantly press the volume key again until you hear 2 beeps

5.recharge power to the headset for a few seconds

6.the headset will turn off and returns to the original setting mode, all of previous pairing data are cleared.

Warning

◊Headset

1.please turn off this headset before put it into your pocket or bag. if it doesn't turn off and press multifunction button accidentally, this headset may dial out with unknow.

2.do not expose the headset under liquid, moisture or humidity as it is not waterproof

3.do not expose the headset under high or low temperature

4.do not expose the headset to contact with sharpness objests as this will cause scratches and damage

5.do not stick anything on inside of the headset as this may damage internal components.

6.do not attempt to disassemble the headset as it does not contain serviceable components.

7.if you do not use the headset to long periods, be sure to store it in a dry place and far away from extreme temperature and dust

◊Built-in battery

1.do not attempt to replace the headset's battery by yourself as it is built-in and is not changeable.

2.only use charger of manufacturer supplied when charging the headset

3.do not disassemble the charger as it may expose you to dangerous voltages or other risks, incorrect reassembly can cause electric shock when the headset is subsequently used

4.do not use the charger in outdoors or under humid environment

5.avoid charging the headset in extremely high or low temperature